^

MATERIAL SAFETY DATA SHEET

I. PRODUCT IDENTIFICATION

Manufacturer

Address:

WD-40 Company

1061 Cudahy Place (92110) P.O. Box 80607 San Diego, California 92138-0607

Telephone:

Emergency only:

Information: Chemical Name: Trade Name:

1-(800) 424-9300 (CHEMTREC) (619)275-1400 Organic Mixture WD-40 Aerosol

II. HAZARDOUS INGREDIENTS

| Chemical Name | CAS Number | % | Exposure Limit ACGIH/OSHA |
| --- | --- | --- | --- |
| Aliphatic Petroleum Distillates Petroleum Base Oil LVP Hydrocarbon Fluid Carbon Dioxide | 8052-41-3 64742-65-0 64742-47-8 124-38-9 | 45-50 15-25 12-18 2-3 | 100 ppm PEL 5 mg/M3 TWA (mist) 1200mg/M3TWA 5000 ppm PEL |
| Non-hazardous Ingredients |  | < 10 |  |

III. PHYSICAL DATA

| Boiling Point: Vapor Density (air=1 ): Solubility in Water: Specific Gravity (HzO=1 ): Percent Volatile (volume): | 323°F (minimum) Greater than 1 insoluble 0.832 @ 72"F 74% | Evaporation Rate: Vapor Pressure: Appearance: Odor: VOC: | Not determined 110±5PSI@70°F Light amber Characteristic odor 412 grams/liter (49.5%) |
| --- | --- | --- | --- |

IV. FIRE AND EXPLOSION

Flash Point:

Flammable Limits:

Extinguishing Media:

Special Fire Fighting Procedures:

Unusual Fire and Explosion Hazards:

131 °F Tag Closed Cup (Solvent Portion) [Lei] 1.0% [Uel] 6.0% CO2, Dry Chemical, Foam. Contents Under Pressure FLAMMABLE - U.F.C. level 3 AEROSOL

V. HEALTH HAZARD / ROUTE(S) OF ENTRY

Threshold Limit Value Symptoms of Overexposure

Inhalation (Breathing):

Skin contact:

Eye contact:

Ingestion (Swallowed):

Aliphatic Petroleum Distillates (Stoddard Solvent) lowest TLV (ACGIH 100 ppm.)

May cause anesthesia, headache, dizziness, nausea and upper respiratory irritation.

May cause drying of skin and/or irritation.

May cause irritation, tearing and redness.

May caused irritation, nausea, vomiting and diarrhea.

First Aid Emergency Procedures

Ingestion (Swallowed):	Do not induce vomiting, seek medical attention.

Eye Contact:	Immediately flush eyes with large amounts of water for 15 minutes.

Skin Contact:	Wash with soap and water.

Inhalation (Breathing):	Remove to fresh air. Give artificial respiration if necessary.

If breathing is difficult, give oxygen.

DANGER! Aspiration Hazard:

Suspected Cancer Agent

Yes        No X

Pre-existing medical conditions such as eye, skin and respiratory disorders may be aggravated by exposure.

If swallowed, can enter lungs and may cause chemical pneumonitis, Do not induce vomiting. Call Physician immediately.

The components in this mixture have been found to be noncarcinogenic by NTP, IARC and OSHA

VI. REACTIVITY DATA

Stability:

Conditions to avoid:

Incompatibility:

Hazardous decomposition products:

Hazardous polymerization:

Unstable

Stable   X

NA

Strong oxidizing agents

Thermal decomposition may yield carbon monoxide and/or carbon dioxide.

May occur		Will not occur X

VII. SPILL OR LEAK PROCEDURES

Spill Response Procedures

Spill unlikely from aerosol cans. Leaking cans should be placed in plastic bag or open pail until pressure has dissipated. Waste Disposal Method

Empty aerosol cans should not be punctured or incinerated; bury in land fill. Liquid should be incinerated or buried in land fill. Dispose of in accordance with local, state and federal regulations.

VIII. SPECIAL HANDLING INFORMATION

Ventilation:

Respiratory Protection:

Protective Gloves:

Eye Protection:

Other Protective Equipment:

Sufficient to keep solvent vapor less than TLV.

Advised when concentrations exceed TLV.

Advised to prevent possible skin irritation.

Approved eye protections to safeguard against potential eye contact, irritation or injury.

None required.

IX. SPECIAL PRECAUTIONS

Keep from sources of ignition. Avoid excessive inhalation of spray particles, do not take internally. Do not puncture, incinerate or store container above 120°F. Exposure to heat may cause bursting. Keep can away from electrical current or battery terminals. Electrical arcing can cause burn-through (puncture) which may result in flash fire, causing serious injury. Keep from children.

X.TRANSPORTATION DATA (49 CFR 172.101)

Domestic Surface

Description: Hazard Class: ID No: Label Required:

Consumer Commodity

ORM-D

None

Consumer commodity (ORM-D)

XI. REGULATORY INFORMATION

All ingredients for this product are listed on the TSCA inventory.

SARA Title III chemicals:	None

California Prop 65 chemicals:	None

CERCLA reportable quantity:	None

RCRA hazardous waste no:	D001 (Ignitable)

SIGNATURE:         R. Miles

TITLE:

Technical Director

REVISION DATE:

NOVEMBER 2003

SUPERSEDES:

NA: Not applicable    NDA: No data available

< = Less than    > = More than

We believe the statements, technical Information and recommendations contained herein are reliable. However, the data Is provided without warranty, expressed
or Implied. It Is the user's responsibility both to determine safe conditions for use of this product and assume loss, damage or expense, direct or consequential,
arising from Its use. Before using product, read label.	ncnc a